HC-21 WiFi-serial communication module User manual V1.2

Product features

◇ 34PIN stamp hole interface by default, set aside two-row pin-type interface pad holes (not welded pins by default):HDR254M-2X4

Support for baud rate range: 1200~460800bps

◇ Support hardware RTS/CTS flow control

◇ Single-3.3V power supply, can offer no less than 300mA DC power supply

Wireless Support for IEEE802.11b/g wireless standard

• Supports two types of wireless networks: Foundation Network (Infra) and  ad hoc networks (ad-hoc)

• Supports soft AP, supports a maximum of 4 STA connection

◇ Supports a variety of authentication mechanisms:

OPEN/WEP/WPA-PSK /WPA2-PSK

Support for fast networking (specify channel and BSSID)

• Supports wireless roaming

Support network created automatically if it does not

Other

Support a variety of network protocols:

TCP/UDP/ARP/ICMP/DHCP/DNS/HTTP/IGMP/PPPOE

◇ Supports DHCP Server and DNS Server

• Supports automatic and orders two working modes

Serial port support transparent transmission mode

• Supports the AT+ controlling instruction set

Support multiple parameter configuration mode:

Com/Web server/Wireless Configuration tab

◇ Built-in friendly Web configuration page

Areas of application

◇ Intelligent appliances, smart home

• Health monitoring, smart toys

• Automotive electronics and industrial control

• Smart grids, the Internet of things

Profile

HC-21 (S2W-M02) is to Exchange technology to launch the new third-generation embedded Simple-WiFi module products.

Simple-WiFi is based on WiFi wireless network-compliant embedded UART interface module, built-in IEEE802.11 wirelessnetworking protocol stacks and the TCP/IP protocol stack to achieve user conversions between serial data to the wirelessnetwork. Through the module, a traditional serial port devices can also be easily connected to a wireless network.

Also, HC-21 (S2W-M02) module except as STA access traditional of Infra network and Ad-hoc network outside, this module also support soft AP mode, NIC work in AP mode Shi can provides smart phone, handheld terminal of access function, while for improved AP mode Xia of user experience, this module in achieved soft AP of while achieved has DHCP Server and DNS Server, to provides smart terminal of zero configuration access, but needs note of is, This module implements DNS Server onlyto provide domain name resolution functionality of this module.

Compared with the previous two generations of products, HC-21 (S2W-M02) in terms of hardware and software has beenfully upgraded, more powerful, more simple to use:

• Transparency in overall better serial data transmission mode, the actual realization of serial port Plug and play

• Less power consumption: 560mW (typical)

• Smaller size: 37mmx23mm

◇ Providing server software, application software, mobile APP design customization services, application software platformenables flexible alternate

1 product description

11 technical specifications

12 hardware introduction

121 mechanical size

122 external interfaces

123 LED indicators Note

124 electrical connection precautions

125 external antenna

126 product design guidelines

13 argument sets

131 serial set

132 pages set

133 SmartConfig set

134 AT command configuration settings

135 Sets the remote UDP

14 precautions for use

2 functional description

21 the network topology

211 ad hoc wireless networks (ad-hoc)

212 based wireless networks (Infra)

22 security

23 fast networking

24 address

25 Wireless roaming

26 TCP/IP protocol stack

3 Quick Guide

31 test platform for quickly building

311 to set up principles

312 platform

313 transceiver test

3131 default parameter test

3132 configuration parameter test

3133 mobile platform test

32 programming principles

331 serial port programming

332 network-side programming

4 typical application examples

41 active serial device networking

42 passive serial device networking

43 radio serial device networking

44 wireless virtual serial port

45 More than network connection mode

46 soft AP mode

5 factory default settings

Appendix

1. product introduction

1.1 technical specifications

Table 1-1 technical product specification

| Free Line | Project | Parameters |
| --- | --- | --- |
|  | Wireless standards | IEEE802.11 b/g 802.11n forcast |
|  | Frequency range |  |
|  | Antenna interface | IPX mini interface/onboard antenna PCB antenna |
|  | Modulation mode | DSSS，OFDM，DBPSK，DQPSK，CCK，QAM16/64 |
|  | Data rate | 802.11b：1，2，5.5，11 Mbps 802.11g：6，9，12，18，24，36，48，54 Mbps |
|  | Transmission power | 802.11b:17 ± 2 dBm (typical) 802.11g:15 ± 1 dBm (typical) |
| Hard Parts | Interface type | 3.3V TTL-level standard UART interface |
|  | Interface number rate | 1200bps ~ 460800bps |
|  | Working voltage | 3.3±0.3 V |
|  | Operating current | 170mA (typical) |
|  | Operating humidity | 5%~90% (no condensation) |
|  | Storage temperature range | -40 ¡æ ~+85 ¡æ |
|  | Working temperature | -20 ¡æ ~+70 ¡æ |
|  | Overall dimensions | 37mm×23mm |
| Soft Parts | Network type | Infra / Ad-hoc / AP |
|  | Security system | WEP / WPA-PSK / WPA2-PSK |
|  | Encryption type | WEP64 / WEP128 / TKIP / CCMP(AES) |
|  | Serial commands | AT+ instruction set |
|  | Network protocols | TCP/UDP/ARP/ICMP/DHCP/DNS/HTTP/IGMP/PPPOE |
|  | Socket connection | The maximum number of connections: 15 |
|  | TCP connection | The maximum number of connections: 8, Maximum number of Client: 8, Maximum number of Server: 3~4, The maximum number of Access Client-side Server: 4 |
|  | UDP connections | The maximum number of connections: 5~6 |
|  | Parameter configuration | Supports up to 4 parameters configuration mode:, A: UART interface commands (AT command mode), Way two: SmartConfig one-touch mode, Way three: Web page parameters configuration, Method four: Udp remotely |

Wireless standards|IEEE802.11 b/g 802.11n forecast

Frequency range|2.412 GHz ~ 2.484 GHz

Antenna interface|IPX mini interface/onboard antenna PCB antenna

Modulation mode|DSSS，OFDM，DBPSK，DQPSK，CCK，QAM16/64

Data rate|802.11b：1，2，5.5，11 Mbps

|802.11g：6，9，12，18，24，36，48，54 Mbps

Transmission power|802.11b:17 ± 2 dBm (typical)

|802.11g:15 ± 1 dBm (typical)

Receiver sensitivity|802.11b:-90 dBm @ 11Mbps (typical)

|802.11g:-70 dBm @ 54Mbps (typical)

Interface type|3.3V TTL-level standard UART interface

Interface number rate|1200bps ~ 460800bps

Working voltage|3.3±0.3 V

Operating current|170mA (typical)

Operating humidity|5%~90% (no condensation)

Storage temp range|-40 C ~+85 C

Working temperature|-20 C ~+70 C

Overall dimensions|37mm×23mm

Network type|Infra / Ad-hoc / AP

Security system|WEP / WPA-PSK / WPA2-PSK

Encryption type|WEP64 / WEP128 / TKIP / CCMP(AES)

Serial commands|AT+ instruction set

Network protocols|TCP/UDP/ARP/ICMP/DHCP/DNS/HTTP/IGMP/PPPOE

Socket connection|Maximum number of connections: 15

TCP connection|Maximum number of connections: 8

|Maximum number of Client: 8

|Maximum number of Server: 3~4

|Maximum number of Access Client-side Server: 4

UDP connections|Maximum number of connections: 5~6

Parameter config|Supports up to 4 parameters configuration mode:

|A: UART interface commands (AT command mode)

|Way two: SmartConfig one-touch mode

|Way three: Web page parameters configuration

|Method four: UDO remotely

HC-21 (S2W-M02) the mechanical dimensions of the module as shown in the following figure:

This product provided 1-3 interface schematic diagram shown in the double dip 8-pin interface pins and stamps of the 34-pin-hole and pin functions defined as shown in table 1-2

1.2.2 external interface

This product is provided 1-3 interface schematic diagram shown in double dip 8-pin interface pins and stamps of the 34-pin-hole and pin functions defined as shown in table 1-2:

Table 1-2 external interface module

| 1 | GND |  | I |
| --- | --- | --- | --- |
| 2 | SPI_SI |  | I |
| 3 | SPI_SO |  | O |
| 4 | SPI_SCK | I | SPI interface clock input (requires additional firmware support) |
| 5 | SPI_CS |  | I |
| 6 | ISP |  | I |
| 7 | GPIO0 |  | I/O |
| 8 | GPIO1 |  | I/O |
| 9 | VCC |  | I |
| 10 | NC |  |  |
| 11 | GPIO2 |  | I/O |
| 12 | SPI_INT |  | I/O |
| 13 | GPIO3 |  | I/O |
| 14 | NC |  |  |
| 15 | TXRX |  | O |
| 16 | NC |  |  |
| 17 | GND |  | I |
| 18 | GND |  | I |
| 19 | NC |  |  |
| 20 | VCC |  | I |
| 21 | UART2_CTS | I | UART2 hardware flow CTS control (not supported) |
| 22 | UART2_RTS | O | Hardware flow control is the UART2 RTS (not supported) |
| 23 | UART2_RX | I | The UART2 data input (not supported) |
| 24 | UART2_TX | O | UART2 data output (not supported) |
| 25 | UART1_TX | O | UART1 data output |
| 26 | UART1_RTS | O | UART1 hardware flow control RTS |
| 27 | UART1_RX | I | UART1 data input |
| 28 | UART1_CTS | I | UART1 hardware flow control CTS |
| 29 | RS485 |  | O |
| 30 | CONFIG | I | Select wireless configuration mode (SmartConfig), active low |
| 31 | RESTORE | I | Select restore factory value input/configuration mode, active low. Restore factory value: the modules in the course of normal operation, the pin input low level, the module will restore Back factory values. Configuration mode: the pin input low level in this case, the module back to electricity, the module into the The AP (configuration mode), smart terminal, will be able to scan to the SSID for Simple WiFi existed. |
| 32 | LINK |  | O |
| 33 | NRST |  | I |
| 34 | GND |  | I |
| 35 | UART1_RTS | O | UART1 hardware flow control RTS |
| 36 | UART1_CTS | I | UART1 hardware flow control CTS |
| 37 | LINK |  | O |
| 38 | NRST |  | I |
| 39 | GND |  | I |
| 40 | UART1_TX | O | UART1 data output |
| 41 | UART1_RX | I | UART1 data input |
| 42 | VCC |  | I |

1	GND		I	Grounding

2	SPI_SI		I	SPI interface data input (requires additional firmware support)

3	SPI_SO		O	SPI interface data output (requires additional firmware support)

4	SPI_SCK	I	SPI interface clock input (requires additional firmware support)

5	SPI_CS		I	SPI interface chip select input (requires additional firmware support)

6	ISP		I	ISP module input (download mode selection, active high)

7	GPIO0		I/O	General purpose input and output

8	GPIO1		I/O	General purpose input and output

9	VCC		I	3.3V power input

10	NC

11	GPIO2		I/O	General purpose input and output

12	SPI_INT		I/O	SPI interface interrupt (falling edge) (requires additional firmware support)

13	GPIO3		I/O	General purpose input and output

14	NC

15	TXRX		O	Output serial port status LED (requires additional firmware support)

16	NC

17	GND		I	Grounding

18	GND		I	Grounding

19	NC

20	VCC		I	Power supply

21	UART2_CTS	I	UART2 hardware flow CTS control (not supported)

22	UART2_RTS	O	Hardware flow control is the UART2 RTS (not supported)

23	UART2_RX	I	The UART2 data input (not supported)

24	UART2_TX	O	UART2 data output (not supported)

25	UART1_TX	O	UART1 data output

26	UART1_RTS	O	UART1 hardware flow control RTS

27	UART1_RX	I	UART1 data input

28	UART1_CTS	I	UART1 hardware flow control CTS

29	RS485		O	485-bus transceiver control pins

30	CONFIG	I	Select wireless configuration mode (SmartConfig), active low

31	RESTORE	I	Select restore factory value input/configuration mode, active low.

Restore factory value: the modules in the course of normal operation, the pin input low level, the module will restore Back factory values.

Configuration mode: the pin input low level in this case, the module back to electricity, the module into the The AP (configuration mode), smart terminal, will be able to scan to the SSID for Simple WiFi existed.

32	LINK		O	Module status LED, output

33	NRST		I	Modular hardware reset input (active low);

Note: after restarting, use AT+Z. Prohibit the use of a hardware Reset to restart.

34	GND		I	Grounding

35	UART1_RTS	O	UART1 hardware flow control RTS

36	UART1_CTS	I	UART1 hardware flow control CTS

37	LINK		O	Module status LED, output

38	NRST		I	Modular hardware reset input (active low)

39	GND		I	Grounding

40	UART1_TX	O	UART1 data output

41	UART1_RX	I	UART1 data input

42	VCC		I	Power supply

1.2.3 the LINK pins LED indicator descriptions

① LED slow Flash 1 time periodically, representing module WiFi is not connected

Periodicity II LED blinks 2 times, represents the WiFi connection is successful, but the network did not connect successfully

③ periodicity LED blinks 3 times, working on behalf of modules are in the configuration mode

④ cyclical rapid flashing LED on behalf of WiFi connection is successful, and the network connection is successful, you can send and receive data

⑤ representatives LED grow light modules are operating restore factory value

1.2.4 electrical connectivity considerations - Used stamps of the 34-pin-hole pin:

1-4 hardware connection diagram figure 1

PIN 6 is the download mode select pin, active high;

Pins 7, 8, 11, 13, for general purpose I/O pin, you can query and set the AT+ directive;

Pin 15 is the serial port status LED output pin (this interface functionality requires additional firmware support);

Pins 25-28 for standard TTL level serial communication;

PIN 29 was 485-bus transceiver control pins;

PIN 30 is used for the wireless configuration mode (SmartConfig), active low;

PIN 31 for configuration mode and select restore factory value, active low;

32 pin connection status LED is output, indicating the module connection status.

When using dual in 8-PIN pin set:

1-5 hardware connection diagram figure 2

• Module supply voltage of DC 3.3V, interfaces as standard serial ports TTL voltage levels.

• Module maximum operating current of 200mA at design should pay attention to meet the module power consumption power supply circuits.

◇ NRST pins can be suspended, but it is recommended to set this PIN to connect to the MCU, so that if necessary, reset the module.

Note: pin 39 ~ 42 for standard TTL level serial communication, PIN pin 35 to 38 for optional features.

Without using multi-function pins dangling under the circumstances, and does not affect the module working properly.

1.2.5 external antenna

HC-21 (S2W-M02) module provides the on-board PCB microstrip antenna and external IPX omnidirectional antenna interface suitable for different application environments. When you use the IPX external Omni-directional antenna, use IEEE 802.11-compliant 2.4GHz antenna, antenna parameters as shown in the table 1-3:

1.2.5 external antenna

HC-21 (S2W-M02) module provides the on-board PCB microstrip antenna and external IPX omnidirectional antenna interface suitable for different application environments. When you use the IPX external Omni-directional antenna, use IEEE 802.11-compliant 2.4GHz antenna, antenna parameters as shown in the table 1-3:

Table 1-3 external antenna matching parameters

Frequency Range (MHz)	2.4GHz ~ 2.5GHz

V.S.W.R				<1.5

Polarization			Vertical

Input Impedance (ω)		50

Joint Connector Type		IPX

Operating Temperature (℃)	-20 C ~+70 C

Storage Temperature (℃)	-40 C ~+85 C

1.2.6 product design guide

Use HC-21 (S2W-M02) module for product design and our sheets need to be aware of the following:

① Annex to module power supply decoupling capacitor should be as close as possible to the module PIN.

II external antenna as far away from metal parts.

If you use the on-board PCB antenna, you should pay attention to:

① below all the levels of the modules, not deposited copper or power-to ensure all PCB layer is clear. In addition, we strongly recommend that other than the entire PCB antenna stretched out across the floor.

II the use of built-in PCB antenna, product cannot use metal.

1.3 parameter settings

Module supports up to 4 parameters configured, now describes configuration of the various parameters.

1.3.1 serial connection settings

Use our tool, can be configured through the serial port on the module.

1. Start serial port configuration tool SimpleWiFi.exe.

2. The WiFi module UART port, through level conversion chip (for example, MAX3232), connected to a COM port on your computer.

3. Click on the "search module", if the module is not properly search, make sure you have properly configured the serial port baud rate, if a module has been configured for automatic working mode of cases, first click on the exiting passthrough mode buttons, as shown in the following figure. After the success shown in the following figure:

4. Depending on where your wireless network wireless router (AP) set the parameters in the parameters change the wireless settings (including network name, encryption, keys, etc) and network settings IP address related parameters;

4. Depending on where your wireless network wireless router (AP) set the parameters in the parameters change the wireless settings (including network name, encryption, keys, etc) and network settings IP address related parameters;

5. Modify the operating mode settings, select the "enable automatic operating mode", and set up the parameters you want to automatically create connections;

6. Parameter after the modifications are complete as shown in the following figure, and click on "submit changes", and in the pop-up dialog box, select the "manual reset";

7. Advanced parameter settings the main design parameters of the serial port to a Simple WiFi module settings.

As shown in the following figure:

Related to user by default you only need to configure the serial port settings.

1.3.2 Web pages (WEB)

Modules can be configured through the parameter page. Module factory default in AP mode, in this mode all supports WiFi 802.11 standard smart terminal can be connected to the HC-21 (S2W-M02) modules and parameters to a module configuration through the browser.

1.3.2.1 configuration page

SimpleWiFi configuration interface through the following 2 ways to enter:

1. HC-21 (S2W-M02) module network (AP/Adhoc) or intranet (Sta) is successful, any smart terminal can be accessed through the WEB page HC-21 (S2W-M02) parameter configuration module.

2. Hardware: in case of short-circuit pins RESTORE power on the module, the module in configuration mode (defaults to AP mode configuration mode). Note that hardware in configuration mode, the module is fixed IP addresses 192.168.2.1. Module configuration mode the parameters fixed, see below.

1.3.2.2 page configuration steps

1. How one computer that is attached to the HC-21 (S2W-M02) module for intelligent end-browser, enter the IP address (the module factory default IP address is 192.168.2.1), as shown in the following figure, a user called Admin. Password is: 123456

2. The navigation bar on the left side of the page, as needed, you can click on the navigation bar into a different setting different pages.

As shown in the figure below: Note: the page is divided into 2 columns, Current currently represents the module parameters. Updated column represents the module parameters in Flash, module reset after use.

Meaning of various page parameters are described below

1-4 Basic Settings the table parameter information

| Parameter name | Values | Parameter meaning |
| --- | --- | --- |
| Band Rate | X bits/second | Serial baud rate |
| Data Size | X bits/character | Serial data length |
| Parity | None | No parity |
|  | Odd | Odd parity |
|  | Even | Even parity |
|  | Mark | Mark parity |
|  | Space | Space parity |
| Stop bits | x bits | Serial number of stop bits |
| Flow control | None/Hardware | Hardware flow control |
| Local Port Number | N/A | This mode is not set |
|  | X | Local communications port, 0 for the local port automatically |
| Remote Port Number | N/A | This mode not set |
|  | X | Remote communication port |
| Mode | Server | Equipment working in the server status |
|  | Client | Equipment working in client status |
| Protocol | TCP | Equipment using the TCP protocol communication |
|  | UDP | Device using UDP protocol traffic |
| Server IP | x.x.x.x | Communication to end IP (x.x.x.x) addresses, you can use a domain name (must be less than 32 characters |
| TCP Link Timeout | x seconds | As TCP Server, if client does not return data within this timeframe disconnect client. 0 means do not time out. |
| Retry connect time | x seconds | Time if the connection is not successful, module will attempt to connect again after this time. 0 represents no attempt. |
| Power Save Mode | Low | Module power-on, based on preset parameters for the connection. If the AP does not exist, will retry in the Retry Connect Time. If the TCP server doesn’t exist, but AP case, try again after Retry Connect Time TCP Connection within time period not disconnected |
|  | Medium | Module power-on, based on preset parameters for the connection If the AP does not exist, or the TCP Server does not exist (any one does not exist), module will disconnect from network Retry the set up Connect Time and try again till successful. |
|  | High | Only when UART data. Module until you reconnect to the network, and TCP. After sending the data, after you set the Retry Connect Time if UART does not send any data, the module will initiate are disconnected from the network, and turn off the RF. |

Table 1-5 WirelessSettings parameter information

| Parameter name | Values | Parameter meaning |
| --- | --- | --- |
| Ssid | 小于32 个字符 | Modules are about to enter the network name (STA/Adhoc) or the module name of the network that will be created (AP) |
| Channel | 1-14/Auto | Channel parameter, 0 for automatic mode |
| NetWork Mode | Infra（STA） | STA mode |
|  | Adhoc | Point to point mode Adhoc |
|  | Infra（Ap） | Ap mode |
|  | AdhocCreate | Reservation |
| Wireless Mode | B/G mode | Wireless mode |
| Rate | Auto/1-54M | Wireless communication rate |
| Encryption | Disabled | Open the encryption mode |
|  | WEP64 | 64-bit WEP encryption mode |
|  | WEP128 | 128-bit WEP encryption mode |
|  | WPA1PSK/WPA2PSK（AUTO） | WPA1PSK/WPA2PSK mode automatically matches the encryption mode |
| Key Format | N/A | This mode not set |
|  | HEX/ASCII | Key format, and 16 hexadecimal or ASCII characters |
| Encryption | N/A | This mode not set |
|  | The encryption key | Encryption keys for encryption mode. Note: WPA encryption mode, the system will automatically convert to HEX mode |
| AP BSSID Filter | Automatic/AP BSSID | Matches the AP MAC address |
| Wireless Roam | Disable/Enable | Turn wireless roaming |

1-6 Advanced Settings the table parameter information

| Parameter name | Values | Parameter meaning |
| --- | --- | --- |
| Address Type | Static IP | Static IP address specified |
| Address Type | DHCP | Dynamic IP addresses |
| Static IP Address | x.x.x.x | IP address |
| SubnetMask | x.x.x.x | Subnet mask |
| Default gateway | x.x.x.x | Default gateway |
| DNS Server | x.x.x.x | DNS server address |
| Data trigger length | x bytes | Data trigger length, serial port when the length parameter specifies the receive data, forcing network packets are sent out on the network. Value range: 32-1024 |
| Login password | xxxxxx | Password must be 6 characters long |
| Restore Factory Defaults | Factory | Restore factory value button |
| System Reboot | Reboot | System reset button Note: after setting all the parameters through Web page, modules MUST BE restarted |

3. After you set all parameters, in the Advanced Settings page after you click the Reboot button, the module will work with set parameters.

1.3.3 SmartConfig mode

This module provides customers with a convenient way to configure wireless, as long as you have a WiFi device, you can configure parameters to a module by this method, just to press the module operation. This functionality needs to be integrated into a user's APP, we offer procedures and algorithms. In addition, our Configuration Manager also provides a demo of SmartConfig. Please refer to the SimpleWiFi serial port configuration tool.

1. SmartConfig mode to enter SimpleWiFi are 2 ways you can enter SmartConfig mode.

| Entry mode | Enter method | Entry mode |
| --- | --- | --- |
| Hardware mode | In the case of shorted pins CONFIG module is powered on | Hardware mode |

2. Enter SmartConfig mode, the program to configure the module can be configured through simplewifi management. Select wireless configuration mode. As shown in the following figure:

3. Click on the settings button, the dialog box appears as shown in the following figure. Please enter through that network card wireless configuration the modules. Note: must use a wireless network card. Then, enter the system password (Web login password), you need to connect the wireless router's network name SSID and password. Click OK then.

Then, click the search module after about 30s, all of the parameters that can be read into the module.

Note: there may be configuration wireless configuration methods are unsuccessful, please reconfigure.

4. After completing the above configuration, click the search module.

1.3.4 AT configuration mode of instruction

Users can send AT command via the UART interface module parameters to a module configuration, see detailed configuration AT instructions.

1.3.5 Configuring remote UDP

WiFi module opens a UDP listener port by default, parameter query and configure commands for monitoring network (AT+), all the commands and serial commands. Host computer can create a UDP socket to an AT command-and-control module.

1.4 usage considerations

Power supply 3.3V modules, interface signals 3.3V TTL-level by default, if you use a 5V system, interface signal level conversion must be carried out.

Maximum power consumption 200mA module, users should pay attention to in selecting power supply power consumption of the module.

users when connected electric module the NRST pins considered to best be on the user's CPU for use when a user needs to reset the module.

module start-up time takes roughly 1 second of time, customers need to note during system design.

other outstanding issues, please refer to the detailed instructions.

2. functional description

2.1 network topology

WiFi wireless network consists of two topological forms: basic network (Infra) and ad hoc networks (ad-hoc), to illustrate that the topology of wireless networks first to understand two basic concepts:

• AP: wireless access point, is a wireless network created side, is also the Central nodes of the network. Our home or business using a wireless router is a AP, other wireless terminals interconnected via AP.

• STA: the wireless sites, Terminal is a wireless network. Laptops, cell phones and other wireless devices with Internet access.

2.1.1 ad hoc wireless networks (ad-hoc)

Ad hoc networks (ad-hoc) only made up of two or more STA wireless network structure, STA on the network can communicate directly.

Note: the HC-21 (S2W-M02) module can be used as access to STA to the AP, also serves as an AP access to STA. But when the AP only support data communications between the STA and AP, does not support direct communications between the STA and STA. As shown in Figure 2-1. 2-1 ad-hoc wireless network topology

2.1.2 basic wireless network (Infra)

Foundation Network (Infra) was created by the AP, many STA joined

Consisting of a wireless network. This type of network is characterized by AP

As the hub for the entire network, all of the network data communication via

AP can be forwarded to complete. As shown in Figure 2-2.

2.2 security

This module supports a variety of wireless network encryption, can fully guarantee the

The secure transmission of user data, including:

• WEP64 2-2 based wireless network topology

◇ WEP128

◇ WPA-PSK

◇ WPA2-PSK

2.3 fast networking

As the STA supports this module to conduct instant networks by assigning a channel number. In the usual process of wireless networking, will first of all current channel automatic scan, to search for the purpose of preparing to connect AP to create (or Adhoc) network. This module provides a set work parameters for the channel, channel conditions where the destination network is known, you can directly specify the working channel of the module, wireless networking time can be reduced from 2 to about 100 milliseconds, so as to achieve the speed of networking purposes.

2.4 Add binding

This module support binding objective in the networking process as a STA network BSSID address (the MAC address) function. Based on the 802.11 Protocol provides that different wireless networks can have the same network name (SSID/ESSID), but it must correspond to a unique BSSID address. Illegal invaders through the method of setting up wireless networks with the same SSID/ESSID, join the STA to illegal AP on the network, causing network leaks. By BSSID address binding way, prevents the STA with access to illegal networks, so as to improve the security of wireless networks.

2.5 wireless roaming

This module as the STA supports wireless roaming functionality based on the 802.11 Protocol. Wireless roaming refers to of is to expanded a wireless network of covered range, by multiple AP common created a has same of SSID/ESSID of wireless network, each AP used to covered different of regional, access to the network of STA can according to by at location of different select a recently (that signal most strong) of AP to access, and with STA of mobile automatically of in different of AP Zhijian switch.

After the open wireless roaming, the module will automatically initiate a scan every 100 seconds, and based on the scan results, select a current AP connection recently. Wireless roaming period, the network may be temporarily disrupted and, therefore, in the case of roaming functionality is not required, and recommended that it be closed.

To note is that after you enable the BSSID address bindings will not be able to use wireless roaming functionality.

2.6 the TCP/IP protocol stack

This module contains a complete TCP/IP protocol stack, and support the TCP/UDP/ARP/ICMP/DHCP/DNS/HTTP/IGMP/PPPOE Protocol also supports SOCKET programming interface based on AT+ instruction, supports DHCP Server/Client to support DNS Server/Resolver, support for HTTP, the built-in WEB server.

3. fast using the wizard

3.1 test platform for quickly building

3.1.1 establishment principle

Module's core function is to convert serial data to TCP/IP data transfer. Hence the modules set up test platforms, you must

Will module serial port connected to the computer serial port module WIFI network-side also needs to establish connections with computers via a wireless network card or wireless router.

Module serial port connected to the WIFI Terminal and computer equipment on the premise that can be obtained through serial debugging tools from the WIFI end

Sending data, but also can be obtained through TCP/UDP tool data sent from the serial port.

3.1.2 platform

Build test platforms need to prepare the following tools, which I provide serial port interface panel and debugging software downloads.

◇ The Configuration Manager: Simple-WiFi configuration management procedures, is provided by our company a fast configuration management module

Procedures used. Used to configure the query/configure, functional testing, and detailed features see section 3.2.

• Serial debugging tools: debugging tools for module serial port data transceiver, ubiquitous gadget on the Web, such as: serial

Assistant Masters, serial port, serial debug, etc. This section uses "serial master" as a demo.

• TCP/UDP tools: debugging tools for module network data transceiver, ubiquitous gadgets on the Web. This chapter

Uses "TCP&UDP test tool" for demonstration purposes.

• PC: used to provide serial port serial port connection with a module. Run the appropriate software to monitor debug serial port and network

Contact-side related data. A simultaneously with wireless network card and the serial output of the PC to complete the test.

◇ Wireless adapters: Desktop host does not have a wireless network card to use with wireless networking capabilities, for connecting modules network port. Tong

Frequently used common USB wireless adapter on the market.

• Serial adapter plate: TTL level serial data for modules and computer serial data RS232 level conversion. Board

There is DC 5V DC power supply, module via the MAX232 chip conversion level after connecting to the PC serial port.

• Serial port extension cable: used to connect the serial port on a computer motherboard DB male head with serial ports DB on the interposer board. The market common.

Note ①: be aware UART serial port contains a TTL level serial and RS232 level serial interface. Serial connection, the module TTL level serial port 3.3V, computer-+12V ~ -12V RS232 levels. Lack of direct communications between the two, MAX232 chip is required after the conversion level, can communication port development board is used here. Users can make the serial port interface panel.

3.1.3 transceiver test

3.1.3.1 default parameter test

This section details without any action at the module under the premise, that is, using the module to the factory default settings, rapid implementation of module to send and receive test. Module plugs into the serial port on the adapter plate, follow these steps.

Step a: the tool will build a platform to prepare according to their respective functional connections as well.

Step b: module default AP mode, launched a similar router function called "SimpleWiFi_xxxx" of WIFI signals, search using a wireless network adapter to the WIFI signal connection.

C: ①the "serial master" software. Select computers using the COM port, baud rate "115200", data bits "8",

Parity bit, "NO", stop bits "1" is the default setting.

② "advanced options" button in the "flow control" dropdown box, select "hardware RTS/CTS flow control", to prevent data loss.

③ Click "opens serial port". You can proceed to the next step.

Step d: open "TCP&UDP test tool" software. Follow the steps above,

① Click ① region shown in Figure 3-1 "creating a connection" popup creates the connection dialog box.

II II the area to fill in the parameters shown in Figure 3-1. Select the type "TCP", fill in destination IP "192.168.1.100", and

Port "50000". Parameters for the module to the factory default settings here can be temporarily ignored. Click the "create" button.

③ hits Figure 3-1 in three parts, as shown in the "create" button to create the connection.

④ in the "sent" fill in the data you want to send.

⑤ Click ⑤ in Figure 3-1 shows a "send" button, the data will be sent over the network module serial port.

At this point in the "serial port Maestro" reception area, you can see "TCP&UDP test tool" sent the data.

3.1.3.2 configuration parameter test

This section details do not have wireless network connection, and test cases with known WIFI signal, and quickly realize the module to send and receive test. Module plugs into the serial port on the adapter plate, follow these steps.

Step a: the tool will build a platform to prepare according to their respective functional connections as well.

Step b: ①the "Simple-WiFi Configuration Manager" in the "serial port" select COM port of the computer, the "settings" confirm baud rate "115200", data bits "8", parity bit "invalid check", stop bits "1".

II click the "exiting passthrough mode" button. Exit successfully, click on "modules".

C: as shown in Figure 3-2, set the "mode" on the basis of network-STA,

Fill in the location of WIFI username and password, such as the company's WIFI

An account number and password. Notice must be in safe mode and launches WIFI signal

Corresponds to the router.

Step d as shown in Figure 3-3, set the "protocol type" TCP, "c/s

Mode "for the client. Server address to fill in open "WIFI TCP& 3-2 set module access parameters

UDP test tool "computer IP, the author's computer IP

"192.168.1.11". Port set, and next steps

"TCP&UDP test" setting corresponds to the port number that is

Can be. Here the port number is set to "60000".

Step e: after confirmation, click on the "submit changes". Pop-up confirmation to restart. Figure 3-3 server parameters settings module

Pop did not respond. Click to confirm.

F: If safe mode did nothing wrong in the wireless settings, the module is connected

To the wireless router. Through the "Simple-WiFi configuration management process

Order "in the" function test "option under" scan "function confirmation is

Whether the normal connection. As shown in Figure 3-4.

G: open the "serial." 3.1.3.1 in step c.

H: open "TCP&UDP testing tools", click "create service

"Enter the native port" 60000 "is set in step d

The port value to module. Click the "Start Server". Will appear

Interface shown in Figure 3-5. Figure 3-4 make sure the module connectivity

Schematic diagram of 3-5 TCP&UDP test tool after you successfully create a connection to send and receive

At this point in the "TCP&UDP test tool" and "serial master" sent and the recipient can send and receive data. The realization of bi-directional communication functionality, and can also be seen, both ends of the module is the transparent transmission data content has not changed.

3.1.3.3 mobile platform

Users can phone platform to data transceiver module testing. Using the mobile phone "TCP&UDP test" server, create the server address and port. Such as "127.0.0.1:50000". In the server address and port number, fill in the module configuration, commit the changes, restart after the module to PC serial port master, receiving mobile phone "TCP&UDP test tool" sent the data. Principle of step 3.1. section 3.2 are largely similar, not repeat them.

3.2 programming works

3.2.1 serial port programming

Standard TTL serial port is a serial protocol-level communication. As this article repeatedly to repeat the module in automatic mode, sending data is inside the module automatically handles end of the transfer agreement to the WIFI network. Automatic processing of the users do not need to bother, only module as an ordinary serial port device. Serial protocol code to send and receive data on a variety of platforms, abound on the Web. The company has no need to design any development protocol packets or DOME program.

3.2.2 network-side programming

Network port is standard TCP/IP packet protocol, this Protocol is responsible for data processing. WIFI network is responsible only for the network connection, TCP/IP packet protocol itself remained associated with WIFI network. Users can communicate via the OCX or API function programming, such as Winsock.ocx. Network communications using either TCP or UDP Server,TCP Client works, works can be set within a module, just note the software end corresponding to the module associated. As Server corresponding to Client,UDP for UDP. Network knowledge to learn on their own.

4. typical application examples

Proactive 4.1-serial device networking

Active serial device networking was initiated by the device connected

And interact with the back-end data server (upload or download).

Typical active type devices, such as wireless POS machines in every credit card

Easy to connect back-end server after the complete and upload transaction data.

Active serial device networking topology is shown in Figure 4-1. Its

In the back-end server as a TCP Server-side device via wireless

AP/router connected to the network, and TCP Client port.

4.1.1 module references settings in Figure 4-1 active serial device networking

Wireless Setup: enter the wireless network name and password for the access point/router

Network settings: enables DHCP to automatically obtain a network address

Advanced BIOS features Setup: enable automatic working modes

Protocol type: TCP server address: backend server IP address

C/s mode: the client port: server port number in the background, such as 6000

4.2 passive serial device networking

Passive serial device networking is, all devices in the system

Always the passive waiting connection state only by the back-end server active

Initiate connection with device and request or download the data.

Typical applications, such as wireless sensor networks, each one

Ends always in real time in collecting data, but the data collected did not

Upload right away, but temporarily saved in the device. Then the server

Periodic Active connectivity devices at regular intervals, and requests to upload or

To download data. At this point, the backend server is actually as a TCP Client

Side, and equipment was as a TCP Server-side.

Figure 4-2 passive serial device networking

4.2.1 module references settings

Wireless Setup: enter the wireless network name and password for the access point/router.

Network settings: IP address: 192.168.0.X subnet mask: 255.255.255.0

Gateway address: 192.168.0.1 DNS address: 192.168.0.1

Mode settings: enabled automatic working modes

Protocol type: TCP client/server mode: the server

Port: 6000 connect timeout: 120

4.3 radio serial device networking

Broadcast serial device to a network, is a network based on UDP protocol, the controlling terminal in the network in the UDP broadcast mode, all of the device-side work in the UDP unicast mode. This approach is essentially a passive network. Most typical applications of traditional RS422/485 wireless network upgrade is connected to the bus network, this network based on UDP broadcast, all devices in the system and system control terminal of the device does not need to make any software changes, use this module to replace the original RS422/485 the hardware directly connected to true "plug and play".

4.3.1 module references settings

Device control port

Wireless Setup: enter the wireless network name and password for the access point/router.

Network settings: IP address: 192.168.0.Y

Subnet mask: 255.255.255.0

Gateway address: 192.168.0.1

DNS address: 192.168.0.1

Mode settings: enabled automatic working modes

Protocol type: UDP

C/s mode: broadcast

Port: 6000

Device x

Wireless Setup: enter the wireless network name and password for the access point/router.

Network settings: IP address of 192.168.0.X

Subnet mask: 255.255.255.0

Gateway address: 192.168.0.1

DNS address: 192.168.0.1

Mode setting: enable automatic working mode 4-3 broadcast serial device networking

Protocol type: UDP

C/s mode: unicast

Port: 6000

4.4 Wireless virtual serial port

4.4.1 module references settings

Unit 1

Enable "network is automatically created if it does not"

Wireless Setup: network type: ad-hoc network

Network name: my_adhoc

Encryption type: open

Network settings: IP address: 192.168.0.1

Subnet mask: 255.255.255.0

Gateway address: 192.168.0.1

DNS address: 192.168.0.1 4-4 Wireless virtual serial port

Mode settings: enabled automatic working modes

Protocol type: TCP client/server mode: the server

Port: 6000 connect timeout: 0

Unit 2

Wireless Setup: network type: ad-hoc network name: my_adhoc

Network settings: IP address: 192.168.0.2 subnet mask: 255.255.255.0

Gateway address: 192.168.0.1 DNS address: 192.168.0.1

Mode settings: enabled automatic working modes

Protocol type: TCP client/server mode: the client

IP address: 192.168.0.1 port: 6000

More than more than 4.5 network connection mode

As shown in Figure 4-5 based on this module is a network topology map of the network connectivity, this system is not designed for a specific application, is not practical, but only as a simulation system for a description. The systematic equipment control module operating in command mode, controlled by the user by entering the AT+ command, which also established four TCP connection, including 2 2 Server and Client. And each Server there are a number of Client (the device) connection. Through the different socket connections, device control can be used with all equipment and servers as shown in the figure for communication.

Figure 4-5 multiple network connection mode

4.5.1 module references settings

Device control port

Wireless Setup: enter the wireless network name and password for the access point/router.

Network settings: IP address: 192.168.0.X subnet mask: 255.255.255.0

Gateway address: 192.168.0.1 DNS address: 192.168.0.1

Mode setting: do not enable the automatic working mode

4.6 soft AP mode

As shown in Figure 4-6 is a module as the AP network topology map, this system is not designed for a specific application, is not practical, but only as a simulation system for a description. Dang this module work in AP mode Shi, up can access 4 a smart terminal, Terminal of IP address can through this module by provides of DHCP Serve module for dynamic distribution, also can static set, this module DHCP Server by distribution of IP address rent period unified for 2 hours, smart terminal to timing of for renewal, smart terminal and this module established wireless connection Hou can for user data of communications, this module work in AP mode Xia Shi, user data communication way with STA mode Xia, As described above.

Figure 4-6 AP mode

4.6.1 module references settings

Wireless Setup: network type: soft AP network name: soft_ap

Encryption type: open SSID broadcast: enable

Network settings: IP address: 192.168.2.1 netmask: 255.255.255.0

Gateway address: 192.168.2.1 DNS address: 192.168.2.1

DHCP Server: open, IP scope IP address for this adapter adds 1 to the MAC address adds 8,

That is, 192.168.2.2 ~ 192.168.2.9, the lease is 2 hours

Module name:

Mode settings: enabled automatic working modes

5. factory default settings HC-21 (S2W-M02) module to factory default settings

|  | Parameter name | Parameter value | Description |
| --- | --- | --- | --- |
| Free Line | Work mode | Basic networking-AP | The other two modes: basic networking-STA, peer networking. |
|  | Network name | SimpleWiFi_XXXX | So here is set, the module will create a file named SimpleWiFi_XXXX (XXXX represents the module last 4 digits of the MAC address), wireless WiFi signals open networks without keys. |
|  | Safe mode | OPEN |  |
| NET Complex | DHCP | Enable | AP mode, DHCP protocol dynamic IP address functionality is enabled by default. Attached SimpleWiFi_XXXX device using a browser at this time, can enter the WEB server configuration module. NIC domain name parameter is valid only in AP mode, the configured domain name less than 32 bytes in length. |
|  | Ip address | 192.168.2.1 |  |
|  | Subnet mask | 255.255.255.0 |  |
|  | Gateway address | 192.168.2.1 |  |
|  | DNS server | 192.168.2.1 |  |
|  | NIC domain name |  |  |
| Workers As Die Type | Automatic working modes | Enable | Enabling automatic work mode, that is, transparent transmission mode. |
|  |  |  | After power up the module's default mode for automatic mode, namely serial transparent transmission mode, so when you configure the module, you need to exit the transparent transmission mode and enter command mode. Use PC-side software search module configuration, when prompted for serial port is in use or the device is not connected, try clicking on "exiting passthrough mode" |
|  | CS models | Server |  |
|  | Server address | - |  |
|  | The port number | 8000 |  |
|  | TCP connection timeout | 120 |  |
| High Class | Created automatically when the network is not present | Enable | The default network is automatically created if it does not enable. |
|  | BG patterns | B/G | Wireless network mode to mixed BG. |
|  | The highest rate of | 54M | Maximum firing rate wireless 54Mbps. |
|  | BSSID | Auto | Not bind BSSID address for your wireless network. |
|  | Channel | Auto | Channel the network of automatic monitoring purposes. |
|  | Wireless roaming | Open | Open wireless network roaming. |
|  | SSID broadcasting | Enable | Enable SSID broadcast, this argument is valid only in AP mode. |
| String Kou | Baud rate | 115200 | Module default baud rate: 115200bps. Serial data format: 8 data bits/1 stop bit/invalid verification no hardware flow control Use PC-side software search module configuration, when prompted for serial port is in use or the device is not connected, you can check the correct baud rate. |
|  | Data bits | 8-bit |  |
|  | Stop bit | 1-bit |  |
|  | Parity bit | Invalid check |  |
|  | Hardware flow control | None |  |
| Penetration Ming Die Type | Automatic frame length | 256 | Automatically set the frame length is 512 bytes. |
|  | Automatic framing cycle | 0 | Automatic framing cycle 0 automatically. |
|  | Escape character | 0x2B | ACSII character "+". |
| The Department of Commission | System password | "123456" | System login password to the string "123456". |
|  | RS485 | Disable | 485 communication transceiver control with system default is off |
|  | Command mode | AT+ | System command mode using the AT+ instruction set. |
|  | GPIO mode | Disable | By default no such GPIO function |

Above the module factory settings, configuration management software can be used to view detailed parameters. View method to refer to the 3rd chapter.

Appendix FAQ and troubleshooting

Q1: the automatic working mode, why always just connected the module can ping Tong, then ping doesn't make sense?

A: it was the start of a power save mode, saving the next time this is the case. Describes in detail refer to the power saving mode.

Q2: Why configure program search module?

A: first, make sure that the baud rate is set correctly. Determine if the module is already in the command line mode, or click on the exit button configuration program.

Q3: module settings WPA AUTO mode why screening is not successful?

A: determine the module at this time did not choose to match the BSSID of the AP, if you choose, you are not able to select AUTO mode of WPA. Remove matching BSSID.

Q4: why did when the module is working in a Sever mode, TCP connection disconnect periodically?

A: WiFi module when you work in the Server mode, if the Terminal Client for a Timeout period is not receiving any data. Module will almost always disconnected from the Client. If the user needs to set the Timeout period to 0., Sever not cyclical at this time disconnected from the Client, but this time it uses WiFi module socket resources.

Appendix FAQ and troubleshooting

Q1: the automatic working mode, why always just connected the module can ping Tong, then ping doesn't make sense?

A: it was the start of a power save mode, saving the next time this is the case. Describes in detail refer to the power saving mode.

Q2: Why configure program search module?

A: first, make sure that the baud rate is set correctly. Determine if the module is already in the command line mode, or click on the exit button configuration program.

Q3: module settings WPA AUTO mode why screening is not successful?

A: determine the module at this time did not choose to match the BSSID of the AP, if you choose, you are not able to select AUTO mode of WPA. Remove matching BSSID.

Q4: why did when the module is working in a Sever mode, TCP connection disconnect periodically?

A:WiFi module when you work in the Server mode, if the Terminal Client for a Timeout period is not receiving any data. Module will almost always disconnected from the Client. If the user needs to set the Timeout period to 0., Sever not cyclical at this time disconnected from the Client, but this time it uses WiFi module socket resources.